# Ajax WaterStop 3/4” (DN 20) Jeweller

Remotely controlled water shutoff valve

| ### Communication | Jeweller communication technology Proprietary wireless technology for events transmission. Features: - Two-way communication. - Protection against device spoofing. - Block encryption featuring a floating key. - Instant notifications. - Remote configuration via Ajax apps. Radio frequency bands 866.0–866.5 MHz 868.0–868.6 MHz 868.7–869.2 MHz 905.0–926.5 MHz 915.85–926.5 MHz 921.0–922.0 MHz Depends on the region of sale. Learn more> Maximum effective radiated power (ERP) up to 20 mW Automatic power adjustment to reduce power consumption and the effect of radio interference. Radio signal modulation GFSK Radio communication range up to 1,100 m up to 3,600 ft Between device and hub (or radio signal range extender) without obstacles. Radio signal encryption Block encryption with a floating key protects all the data stored and transmitted. Frequency hopping Prevents radio interference and jamming.. |
| --- | --- |
| ### Operating components | Shutoff valve ¾" (DN 20) RuB valve Included in the complete set. Electric actuator Opens or closes the valve. Mounting platform To install an electric actuator on a shutoff valve. Mounting lockers To fix an electric actuator on a shutoff valve. The complete set includes two options: - standard (pre-installed) - anti-sabotage |
| ### Shutoff valve | Fields of application water supply and heating systems Operating fluid hot and cold water, non-aggressive liquids Material brass Connection type and thread BSP parallel (EN 10226-1, ISO 228), female-to-female threads NPT taper (ANSI B.1.20.1), female-to-female threads Thread size: ¾" (DN 20, 20 mm) Operating liquid temperature range from -20 °C to +170 °C from -4 °F to 350 °F Freezing of the fluid in the pipeline may severely damage the valve and electric actuator. Mounting platform to mount an electric actuator on a shutoff valve Complies with the ISO 5211 standard. Lever on mounting platform To manually shut off the water supply without tools in case of emergencies. |
| ### Electric actuator | Torque up to 8.5 N•m Operating pressure up to 10 bars ​up to 145 psi Shut-off speed up to 7 seconds The value may increase if the valve is clogged or stuck. Remote control control and configuration in Ajax apps Apps provide all information about the device: current valve state (open/closed), statuses of battery charge, external power supply, and communication with a hub. Manual control the control button on the electric actuator enclosure Opens/closes the valve when pressed. |
| ### Additional features | Installation - does not require tools and enclosure disassembling - 4 possible mounting positions of electric actuator towards the valve - anti-sabotage locker to complicate unauthorized dismount of an electric actuator The design allows maintenance, and repair in hard-to-reach places: pipe tunnels, technical boxes, walls, etc. It is possible to replace the actuator unit without dismantling the shutoff valve. LED indication The LED placed under an Ajax logo on enclosure and reports the electric actuator states. Automation scenarios - alarm reactions - security mode change reactions - scheduled actions - temperature scenarios - humidity scenarios - CO2 level scenarios - by pressing Button - by pressing LightSwitch Overheating notification The notification is sent to Ajax apps when the device temperature tops +60 °C 140 °F and when it returns to normal. |
| ### Recommendations | The device is designed for indoor use only. Contact a plumber to install the shutoff valve. |
| ### Sabotage protection | Protection against spoofing device authentication Communication failure detection in 36 s The time depends on the Jeweller (or Jeweller/Fibra) settings. Tampering alarm Notifies about attempts to detach the enclosure from the mount. Mounting lockers for fastening The complete set includes two options: - Standard locker (pre-installed) — fixes an electric actuator on a shutoff valve. Removes easily for a quick access to the valve. - Anti-sabotage locker — used instead of a standard locker. Securely fixes an electric actuator on the valve to complicate an unauthorized dismount. |
| ### Power supply | Pre-installed batteries 4 × CR123A battery Up to 3 years of battery life. Optional external power supply 7.5–14 V⎓, from 1.8 A The recommended parameters of the external power supply are 9–12 V⎓, 2 A. The batteries serve as a backup when an external power supply is connected. Use a round 2 × AWG22 cable with an external diameter of 3.0–3.7 mm to connect an external power supply. |
| Dimensions | Shutoff valve with an electric actuator 104 × 150 × 70 mm 4.09" × 5.91" × 2.76" Electric actuator 93 × 70 × 95 mm 3.66" × 2.76" × 3.74" Shutoff valve 80 × 32 mm 3.15" × 1.26" Weight 1012 g 35.7 oz Shutoff valve with an electric actuator. 536 g 18.9 oz Electric actuator. 476 g 16.79 oz Shutoff valve. Operating temperature range from 0 °C to +60 °C from 32 °F to 140 °F Operating humidity up to 95% |
| ### Colors | Black White |
| ### Complete set | Ajax WaterStop Jeweller 4 CR123A batteries Pre-installed. DN20 (¾”) RuB valve 2 lockers for securing the electric actuator Sealing nut Quick start guide |
| ### Warranty | 24 months |